November 6, 2021

EE463 Static Power Conversion 1

HOMEWORK-1 REPORT

Zeynep Çöklü,2231694

Mert Eren Kandilli,2304855

# Introduction

In this homework, the simple single-phase half wave and full-wave rectifier, and the three-phase full-wave rectifier will be analyzed. The output voltage waveform will be observed. The average voltage values and the total harmonic distortion values will be calculated and compared with the simulation results. In addition, the effect of the grid side inductance will be observed. The different type of rectifier concepts will be clear after the simulation and observations.

## PART 1 : The Single Phase Half-Bridge Rectifier

Figure 1: The Circuit Schematic for Single Phase Half-Wave Rectifier

- For the part-a, the circuit schematic in above is simulated with different time steps such as 1 ns, 0.1 ms and 1 ms. Note that RL is  80 ohm and AC voltage source generates 230 Vrms at 50 Hz which is the Turkish electricity grid phase neutral voltage. However, the AC source in Simulink requires to giving peak value of voltage. Therefore, 230 Vrms is multiplied with square root of 2. The output voltage waveforms are given in below.

Figure 2: The Output Voltage Waveform for Time-Step 1 ns

Figure 3: The Output Voltage Waveform for Time Step 0.1 ms

Figure 4: The Output Voltage Waveform for Time Step 1 ms

- As can be seen from the figures in part-a, the waveform for time-step 1 ms is distorted. It is because the time step is too large to create a smooth waveform. Since, the output voltage has many harmonics inside, the proper step time is necessary to see the waveform clearly. Other way around, if the step time is too small, the simulation takes too much time and it creates not a good observation for the user. The proper value can be determined by looking the frequency of the voltage. In this case, the best option to simulate is 0.1 ms.

- Calculation of the average output voltage waveform and THD:

Figure 5: The Circuit Schematic to obtain THD and Mean Voltage

- As can be seen from the Figure 5, the average voltage is equal to 103.5 Volts which is consistent with the analitically calculation. In a single phase half bridge rectifier, the mean voltage of the output is roughly equal to 0.45Vrms. In this case Vrms value for the AC voltage source is 230 Volts. Then:

Vmean=2π*Vrms≅0.45*Vrms=0.45*230 V=103.5

## PART 2 : The Single Phase Full-Bridge Rectifier

- In practical applications Ls and Rs on the given circuit refers to internal inductance and resistance value of the AC Voltage source

Figure 6: The Circuit Diagram of Single Phase Full Bridge Rectifier

In part b we are asked to obtain 3% output voltage peak-to-peak ripple voltage to mean ratio by adjusting the capacitor accordingly. In this circuit capacitor carries out filtering operation. When a source is connected to the circuit first capacitor draws current and becomes charged. When AC voltage is lower than the capacitor voltage, then capacitor feeds the load so that we obtain more DC like output voltage. Therefore, capacitance value should be chosen wisely. For example if capacitance value is choosen too large, then in the beginning it draws large amount of current that might harm the diodes. Moreover, if capacitance value is too small, then capacitor will not be enough the feed the circuit, this leads to obtain worse DC like output voltage.

In this diagram all the calculations are shown, and 3% ratio is obtained with C= 3.92 mF.

Figure 7: The Output,Input Voltage and Current of the Single Phase Full Bridge Rectifier with Rs and Ls

As mentioned, in the beginning there is a jump on currents graph due to capacitor. Also we can say that the capacitance value affect the time required for circuit to become stable.

- In part (b), necessary measurements are shown.

Vavg=307.2 Volts

THD of Input Current=1.121

The Power Factor pf≅0.993

Figure 8: The Circuit Diagram of Single Phase Full Bridge Rectifier without Rs and Ls

In part (e), Ls and Rs are removed, that is we assumed that the AC voltage source is ideal.

Figure 9: The Output,Input Voltage and Current of the Single Phase Full Bridge Rectifier without Rs and Ls

When Ls and Rs are removed, we observed that now current have sharp transitions. This is because when there is an inductor in the circuit, it prevent the sharp current transitions because it results in infinite voltage on the inductor. This leads input current to increase suddenly so we can say that charging time of the capacitor (time required for stabilization of the circuit) is decreased. In addition to these, following differences are obtained.

- Average output voltage increased.

- Capacitance value required to have 3% ratio decrased.

- Input current increased and has sharp edges.

- Power factor decreased.

- THD increased.

## PART 3 : The Three Phase Full-Bridge Rectifier

- In the Part-a, the circuit schematic simulated in below figure. The three-phase voltage source is arranged to 400 Vrms and 50 Hz as in Turkey electricity grid. Also, the DC current source is equal to 60 Amps.

Figure 10: The Output Voltage and Input Current Waveforms for Three-Phase Full Bridge Rectifier without Ls

Figure 11: The Circuit Diagram for Three Phase Full Bridge Rectifier without Ls

The output voltage waveform and input current waveform are given in Figure 11. As can be seen, the peak value of the output voltage is equal to 2*Vrms which is roughly equal to 565 Volts. Also, the input current waveform is square waveform as expected. The peak value is 60 Amps like DC current sources.

Unlike single phase rectifier we do not observe a square input current waveform but there are some interval that no input current is supplied.

- In order to find the mean voltage of three-phase full bridge rectifier:

Vavg=62ππ/34π/32*400sin100πtdt=12002π[-cos(100πt)]4π/3π/3

Vavg=12002π≅540 Volts for 400 Vrms

As can be seen from the simulation figure, the mean of output voltage is observed 539.9 Volts which is nearly same the theoretical value.

Figure 12: The Harmonic Analysis of the Input Current (without Ls)

By using the FFT analyzer in powergui, the harmonic analysis is made as follows.

As can be seen from the Figure 12, the majority of harmonic content for the input current waveform is the 5th harmonic. The 7th harmonic follows it. Notice that, the third harmonic has very little effect on the input current which is expected.

Figure 13: The Harmonic Analysis of the Output Voltage (without Ls)

For the output voltage, the harmonic analysis is also performed in Figure 13.

Figure 14:  The Circuit Diagram for Three Phase Full Bridge Rectifier with Ls=1.5mH

The circuit diagram of three-phase full bridge rectifier with Ls=1.mH is given in Figure 14.

As can be seen from the figure, the resistance is connected parallel to DC current source because the Simulink creates error when the inductances are connected the current source directly. Therefore, 1 Mohm resistance is connected parallel to diodes.

Figure 15:The Output Voltage and Input Current Waveforms for Three-Phase Full Bridge Rectifier with Ls=1.5mH

The output voltage and inpur current waveforms of the three phase full bridge rectifier with Ls=1.5 mH are shown in Figure 15 below.

When the source inductances are added to three phase full bridge rectifier, the commutaion or overlap happens. Because of the inductances diodes cannot close immeditely, therefore, in a small period two phases are conducting at the same time.

Because of the commutation, the average voltage decreases with a small amount which is :

∆V=62πωLsId

This equation implies the area lost in output voltage waveform. Another important change in the input current waveform. Because of the differential relation between inductance voltage and current, the current cannot change immeditatly. Therefore, it follows the way in Figure 15.

- The average output voltage with commutation can be calculated as follows :

Vavg=32πVl-l-3πωLsId

The frequency of the frequency of three phase voltage source is 50 Hz which means ω=2πf=100π. Also, the inductance value is 1.5 mH and the DC current is 60 Amps. Hence :

Vavg=32π*400-3π*100π*1.5*10-3*60≅513 Volts

Also, the average output voltage is measured from the simulation program as can be seen from Figure 14. The measured value is 512.8 Volts which is very near to theoritical value.